ACEMID Interim Study - Export Instructions Last updated: 6-Aug-25

Background

The ACEMID interim study will analyse data from the "loose files" that are generated during each Vectra capture. This includes the lesion data JSON files (naevi count and distribution, skin colour), TOM files (height and body surface area), and CR2 images (sun damage assessment).

This document outlines the process of exporting the interim study participants from the clinical Vectra databases, in order to perform analysis on copies of the data in secure research environments. Instructions and scripts for the analysis step will be provided at a later date.

If you have any questions or issues with anything described in this document, contact Adam Mothershaw: a.mothershaw@uq.edu.au

Prerequisites

1. This file

2. The Vectra DB utility: VectraDBTool_v1.8.exe

3. The list of study participants for your site: <site>_id_list.txt

4. The export script: exporter.ps1

5. The export script runner: run_export_script.bat

6. All interim study participants provided with an External ID in the Vectra software.

External ID

The External ID field exists in the Vectra software's patient data form. It is preserved during a "cleansed export", unlike Patient ID which is removed (along with name, DOB, etc). Set each person's external ID to their ACEMID ID used in e.g. REDCap. This allows data linkage and correct analysis after the cleansed export takes place.

7. Secure storage for saving the exported data (allow approximately n × 20GB of storage, where n is the number of participants being exported).

8. Sufficient time for the process to complete (allow 1-5 minutes per participant, depending on the size of their data and the speed of the storage).

Instructions

1. Locate the Vectra application file vectra.exe on the PC. Often it's at: C:\vectra\bin\vectra.exe.

2. Copy VectraDBTool_v1.8.exe into the same folder as the vectra.exe program. 3. Run the VectraDBTool_v1.8.exe file that you just copied into the bin folder, to check that it starts without any errors or complaints. When it opens, you will see a small popup window titled Database list. Select the ACEMID database from the Select a database: drop-down list and click OK.

ACEMID database name

If you don't know the name of your ACEMID database: start the Vectra software, and click the setup button on the bottom-right corner of the main window. A popup window will appear with multiple tabs, select the Database tab and you will see the name of the active database listed next to database name.

4. From here, proceed using either the GUI method or the script method: 1. GUI method: requires you to manually select which patients to export by clicking their name in a list. Time-consuming but easy to use.

2. Script method: requires running a script and typing-in values for some file and folder locations. Less time-consuming overall as it reads in the study ID list from a file.

Choose one of these methods and continue below.

Option 1: GUI method

5. In the main window of the VectraDBTool program, select Tools > Export Patients.

6. The Export Patients window will appear, containing a list of all patients in the selected database. Resize the window so that all the columns are visible, then click the External ID column header to sort by that column.

7. Open the list of study participants for your site, then use that list as a reference while you click to highlight each interim study participant in the Export Patients window. 8. Once all interim study participants have been highlighted, ensure that the checkboxes below the list have the following settings:

1. cleanse patient data: ✅

2. backup images: ✅

3. combine patients: ❌

9. Click Export and a new popup window will appear titled Select a directory to export. This is the target folder to which all the exported data will be saved. Browse to the drive or folder where you want to save the data, and then click Select folder.

10. While the export is running, the cursur within the VectraDBTool window will change to the "busy" icon ( or ). The program does not provide any additional status updates or progress bar, however the process can be somewhat monitored by looking at the destination folder for the exported data: each exported participant will create 1 x .db file and 1 x folder. When the process is finished, a small popup window will appear with the message Patients have been exported.

Option 2: Script method

5. Close the VectraDBTool window.

6. Double-click the file run_export_script.bat. (The export code is actually contained exporter.ps1, however the run_export_script.bat file is required in order to correctly initiate the script).

7. You will be prompted to type in several file and folder paths:

a. Enter full path to VectraDBTool_v1.8.exe: - see Step 2. In most cases, it will be something like c:\vectra\bin\VectraDBTool_v1.8.exe.

b. Enter export directory: - the folder or drive where all the exported data will be saved.

c. Enter full path to interim ID list file (e.g. C:\path\ids.txt): - the txt file containing the list of study IDs for your site.

d. Enter database name: - as determined in step 3 of instructions.

8. After entering the database name, you should see the message Running command... , shortly followed by Press any key to continue (if you press any key at this stage, the window will close but the process will continue in the background). Due to limitations with the export software, it is not possible to show any status or progress information. You will need to open the export directory to see if the script is working (i.e., new files and folders are being created in the export directory). Each exported participant will create 1x .db file and 1 x folder.

Post-export Instructions

Count the number of .db files in the export folder, this should match the number of subfolders that have been created in the export folder, and both these numbers should match the number of participants being exported from your site.

If the number of .db files matches the number of subfolders, however the number is less than expected (i.e. less than the number of interim study participants being exported), either:

1. the export process is still running - wait 20 minutes and see if any additional files/folders are added to the export folder. If no more files are created, then:

2. the export process exited early early due to an unknown reason. Check that there is still storage space available in the export folder, and then re-start the process. If the exporter repeatedly seems to crash at a certain point, this will require further investigation outside the scope of this document.