ACEMID Westmead Data Extraction Manual

Prepared by Yagiz Aksoy | September 2026 |

# 1. What this is

This is the data-extraction procedure for the Westmead ACEMID site, covering two national studies that draw on the Vectra imaging data: the UQ interim analysis and the Monash-led AI Lesion Triage filter paper. It is adapted from the runbook already in use at MIA, so the workflow is proven; the tool quirks discovered there are built into the steps below rather than left to be rediscovered.

The work is organised into phases. The exports that need the Vectra workstation come first; the two Python-based steps and the tile preparation come later and can run on any PC. Everything is resumable, so it does not need to be completed in one sitting.

# 2. Files and tools

## 2.1 Files provided (in the shared folder)

| Filename | Used in | What it is |
| --- | --- | --- |
| WMD_interim_id_list.txt | Phases 2 and 3 | 272 Westmead interim study participants, one ID per line. Paste for every interim export. |
| WMD_filterpaper_id_list.txt | Phase 2 | 5 Westmead filter paper participants. Paste for every filter paper export. |
| lesion_json_export.py | Phase 2, JSON step | Copies the multi-visit lesion JSON files. Copy-only; cannot modify the clinical database. |
| exporter.ps1 + run_export_script.bat | Phase 4 | The de-identified (cleansed) patient export. The two files must sit together. |
| script_2.py + raw_converter_v1 + raw_converter_v2 + readme.pdf | Phase 5 | Tile preparation for the photodamage analysis. Keep all four together in one folder. |

## 2.2 Tools needed on the Vectra workstation

| Item | Detail |
| --- | --- |
| VectraDBTool.exe (Dec 2025 build, v1.11) | In C:\Vectra\bin\. This is the version to use for the lesion analysis, dermoscopy and ISIC exports. See the note below on tool versions. |
| ISIC2024tool.exe | In C:\Vectra\bin\lesion-analysis\ (create the subfolder if it does not exist). |
| Python 3.9 or newer, with the Pillow package | For the JSON step and the tile preparation. Check with: python --version. If not installed, a request to local IT usually resolves it; a portable Python from python.org also works without admin rights. |
| Secure storage | External drives or the site’s preferred secure storage. Allow roughly 3 to 5 TB for the full Westmead cohort (the cleansed export is the large component). |
| ACEMID database name | Vectra > Setup (bottom right) > Database tab. Every tool asks you to select a database at launch; always pick this one. |

Three lessons from MIA, already built into the steps below 1. Use VectraDBTool v1.11, not v1.8. On the current database schema, v1.8 silently produces empty (header-only) CSV files. v1.11 exports correctly. If any export comes out empty, the tool version is the first thing to check. 2. Strip ID suffixes. Study IDs ending in _W, _L or _D do not match in the tool’s MRN box. Remove the suffix when pasting, and keep a note mapping the modified IDs so they can be rejoined later. 3. Watch for invisible whitespace in patient IDs. A few participants may have a blank space before or after their ID inside the Vectra patient record, which causes them to be silently dropped from an export. It is only visible by opening the ID field in Vectra (Open chart > Open patient data). If the export count comes up short, this is the likely cause: remove the space in the record and re-run those participants.

Three rules for every export 1. Never leave the MRN box blank. A blank box exports the entire clinical database, not the study cohort. 2. One list per export, into its own labelled folder. Never mix outputs from the two studies in one folder. 3. The tools look frozen while they work. No progress bar appears. Do not close them. Track progress by watching files appear in the output folder.

# 3. Phase 2: filter paper exports (small, a good place to start)

What this is: the complete Westmead contribution to the AI Lesion Triage filter paper. Only 5 participants, so this is the quickest way to get Westmead on the board and to confirm the toolchain works end to end. Supports: Lesion Triage filter paper.

All steps use WMD_filterpaper_id_list.txt (5 IDs). Paste the whole list, stripping any _W/_L/_D suffixes.

## Step 1: lesion analysis CSVs, ALL lesions

- Run VectraDBTool v1.11. Select the ACEMID database. Menu: DermX Utils > Export lesion analysis data.

- UNCHECK "Export manually tagged lesions only" (this study needs all auto-detected lesions). Output to 01_lesion_csv_filter_all\.

- Paste the filter paper list, Process. Then open two or three CSVs and confirm they contain data rows with both a uuid and a LesionID column. Empty files mean the wrong tool version.

## Step 2: dermoscopy images

- Menu: DermX Utils > Export dermoscopy data. Settings: Include image files ON; D200 colour profile (the one matching how images look in DermaGraphix, not P1/P3 as-captured); dexi OFF.

- Output to 02_dermoscopy_filter\. Paste the list, Process.

## Step 3: ISIC image tiles

- Using the same tool: DermX Utils > Export ISIC 2024 tool data. Output to 03_isic_tiles_filter\. Tick "do not overwrite existing data". Paste the list, OK.

## Step 4: multi-visit JSON files (Python)

- Open lesion_json_export.py, set mapping_lesion_csv_folder to the 01_lesion_csv_filter_all folder from Step 1, output_folder to 04_lesion_json_filter\, and confirm include_all_visits = True. Run: python lesion_json_export.py. Check the manifest it writes for visit counts.

# 4. Phase 3: interim study exports (the main cohort)

What this is: the metadata layer for the UQ interim analysis. The lesion analysis CSVs link each REDCap lesion record to the imaging; the dermoscopy export captures the close-up images for the same cohort. Supports: UQ interim study.

Both steps use WMD_interim_id_list.txt (272 IDs, suffixes stripped).

## Step 1: lesion analysis CSVs, TAGGED ONLY

- VectraDBTool v1.11 > Export lesion analysis data. CHECK "Export manually tagged lesions only" (note: the opposite setting from the filter paper). Output to 05_lesion_csv_interim_tagged\. Paste the interim list, Process. Roughly 25 seconds per participant.

- Some participants will legitimately produce a header-only file because they have no manually tagged lesions. That is correct, not an error; note which ones so they are not later mistaken for failures.

## Step 2: dermoscopy for the interim cohort

- Same settings as the filter paper dermoscopy (images ON, D200, dexi OFF), into 06_dermoscopy_interim\, with the interim list. Run in chunks if needed; already-exported participants are skipped when do-not-overwrite is on.

# 5. Phase 4: cleansed patient export (pilot first)

What this is: the main data capture for the interim analyses. The cleansed export copies each participant’s full imaging record with names, dates of birth and patient IDs stripped out (only the ACEMID study ID remains). It contains the lesion JSONs, TOM files (height, body surface area) and CR2 images (sun damage), and it is the input the Python analysis steps run on. Supports: UQ interim study, and all future analyses on this cohort.

Pilot before the full run The cleansed export is large, roughly 20 GB per participant. Do not start with the full 272. Run a pilot of 10 first, measure the actual size and time, then confirm the batching and storage plan before committing the cohort.

- Copy exporter.ps1 and run_export_script.bat into one folder, double-click the .bat, and answer its prompts: path to VectraDBTool_v1.8.exe (this legacy step uses the export script as provided), the export directory (07_patient_export\), the ID list file, and the database name.

- For the ID list, use a pilot file of the first 10 interim IDs. Each participant produces one .db file plus one folder. When the pilot finishes, note the total GB and the time taken, and confirm the plan for the rest before scaling up.

- Note: some older captures may hold JSON files at an outdated version missing required columns. This can only be fixed on the live Vectra software (open the patient, load the baseline visit, wait for the busy icon to clear, then re-export that participant). If flagged during processing, it must be done before the site’s Vectra access ends.

# 6. Phase 5: tile preparation for the photodamage analysis (script 2)

What this is: preparation of de-identified image tiles for the UQ photodamage analysis. The script reads the cleansed export, converts each participant’s baseline photos to PNG, cuts them into small tiles, and quality-checks each tile, keeping only skin-containing tiles and discarding anything that could show faces or background. The tiles are what transfer to UQ; the raw photos never leave the site. Supports: UQ interim study.

- Setup: keep script_2.py, readme.pdf and the two converter folders together; install Pillow (python -m pip install Pillow); set CLEANSED_DATA to the 07_patient_export folder and output_folder to 08_script2_tiles\.

- Run it over the 10 pilot participants first (python script_2.py, about 4 minutes each), then over each cleansed export batch as it completes. It runs on any PC with the drive attached, so it need not tie up the Vectra terminal.

- Before any tiles are transferred, a sample should be checked to confirm nothing identifiable remains. Please send me the QC summary and a sample from one participant for that check.

# 7. Governance and QC

- All extraction sits within the existing ACEMID approvals (HREC/2019/QMS/57206) and the Westmead / NSW Health data arrangements. Nothing identifiable should leave the site: filenames and folders carry ACEMID study IDs only.

- Counts reconcile: participant folder/file counts in each export match the ID-list count for that run (allowing for participants with no captures).

- Spot-check a sample of CSV contents and image folders for any residual identifiable information before transfer.

- Keep the suffix-mapping note and the header-only-participant note with the outputs, so the central teams can interpret the data correctly.